## Phase 1: Base Operating System Setup

- Install Ubuntu Desktop LTS on the Chromebox.

- During the installation steps, ensure you check the box for "Log in automatically."

- Once booted to the desktop, open Settings → Power and disable Screen Blanking / Automatic Suspend so the screen never goes to sleep.

## Phase 2: System Packages & Firewall Setup

Open a terminal (Ctrl + Alt + T) and paste this block to update the system, open the network ports for AirPlay mirroring, and install the Docker engine:

Bash

# 1. Update the operating system packages

sudo apt update && sudo apt full-upgrade -y

# 2. Open standard firewall ports for UxPlay mirroring & mDNS discovery

sudo ufw allow 5353/udp

sudo ufw allow 7000:7002/tcp

sudo ufw allow 7000:7002/udp

sudo ufw reload

# 3. Install AirPlay streaming dependencies and the Avahi network daemon

sudo apt-get install -y ca-certificates curl avahi-daemon gstreamer1.0-plugins-bad gstreamer1.0-libav gstreamer1.0-vaapi

# 4. Install the official Docker Engine and Compose plugins

sudo install -m 0755 -d /etc/apt/keyrings

sudo curl -fsSL https://download.docker.com/linux/ubuntu/gpg -o /etc/apt/keyrings/docker.asc

sudo chmod a+r /etc/apt/keyrings/docker.asc

echo "deb [arch=$(dpkg --print-architecture) signed-by=/etc/apt/keyrings/docker.asc] https://download.docker.com/linux/ubuntu $(. /etc/os-release && echo "$VERSION_CODENAME") stable" | sudo tee /etc/apt/sources.list.d/docker.list > /dev/null

sudo apt-get update

sudo apt-get install -y docker-ce docker-ce-cli containerd.io docker-buildx-plugin docker-compose-plugin

# 5. Enable Docker to start on system boot

sudo systemctl enable docker.service

sudo systemctl enable containerd.service

# 6. Authorize your user account to manage Docker containers without sudo

sudo usermod -aG docker $USER

⚠️ CRITICAL: You must reboot the Chromebox (sudo reboot) right now before proceeding to Phase 3, or the user permissions won't apply and the following steps will throw permission errors.

## Phase 3: Deploy the Signage Stack (Anthias)

Once rebooted, open your terminal back up and build the local digital signage container engine using our customized, hardcoded settings.

- Create the screenly folder structure:

- Bash

mkdir -p ~/screenly && cd ~/screenly

- Open a new configuration file:

- Bash

nano docker-compose.yml

- Paste this exact configuration block, which uses the correct x86 images, fixes the memory issues, and strips out the broken Raspberry Pi driver dependencies:

YAML

version: '3'

services:

anthias-redis:

image: ghcr.io/screenly/anthias-redis:latest-x86

restart: always

mem_limit: 512m

anthias-server:

image: ghcr.io/screenly/anthias-server:latest-x86

restart: always

depends_on:

- anthias-redis

volumes:

- ./data:/data

ports:

- "80:80"

mem_limit: 512m

anthias-viewer:

image: ghcr.io/screenly/anthias-viewer:latest-x86

restart: always

shm_size: 128m

depends_on:

- anthias-server

environment:

- DISPLAY=:0

volumes:

- /tmp/.X11-unix:/tmp/.X11-unix

mem_limit: 512m

anthias-websocket:

image: ghcr.io/screenly/anthias-websocket:latest-x86

restart: always

depends_on:

- anthias-server

mem_limit: 512m

- Press Ctrl + O, then Enter to save, and Ctrl + X to exit the text editor.

- Launch the containers in background mode:

- Bash

docker compose up -d

## Phase 4: Configure the Firefox Proxy Whitelist

Because the network filters block standard web looping, we must whitelist local playback.

- Open Firefox manually.

- Click the Menu (three lines) in the top right corner → Settings.

- Scroll down to the very bottom of the General tab and click Settings... under Network Settings.

- Find the text box labeled "No proxy for" and add this exact string to it:

- Plaintext

localhost, 127.0.0.1, 192.168.0.0/16, 10.0.0.0/8

- Click OK and close Firefox.

## Phase 5: Automate Screen Mirroring (UxPlay Service)

Create a system service so the AirPlay receiver starts automatically at boot on pinned, unblocked ports.

- Open a terminal file creator:

- Bash

sudo nano /etc/systemd/system/uxplay.service

- Paste this configuration block (replace yourusername with the actual name of your Chromebox Ubuntu user profile):

- Plaintext

[Unit]

Description=UxPlay AirPlay Receiver

After=network.target avahi-daemon.service docker.service

[Service]

Type=simple

User=yourusername

Environment=DISPLAY=:0

ExecStart=/usr/bin/uxplay -p 7000 -fs -vsync no

Restart=on-failure

[Install]

WantedBy=graphical.target

- Save and close (Ctrl + O, Enter, Ctrl + X).

- Register and enable the background hook:

- Bash

sudo systemctl daemon-reload

sudo systemctl enable uxplay.service

sudo systemctl start uxplay.service

## Phase 6: Automate the Fullscreen Kiosk on Boot

Finally, create the startup rule that launches Firefox directly into fullscreen mode after waiting for Docker to finish loading.

- Make sure the hidden user autostart folder exists:

- Bash

mkdir -p ~/.config/autostart

- Create the startup config file:

- Bash

nano ~/.config/autostart/anthias-kiosk.desktop

- Paste this desktop metadata inside:

- Plaintext

[Desktop Entry]

Type=Application

Exec=sh -c "sleep 10 && firefox --kiosk http://127.0.0.1"

Hidden=false

NoDisplay=false

X-GNOME-Autostart-enabled=true

Name=Anthias Signage Kiosk

Comment=Launches local digital signage player after a 10s delay

- Save and close (Ctrl + O, Enter, Ctrl + X).

- Give the startup shortcut executable rights:

- Bash

chmod +x ~/.config/autostart/anthias-kiosk.desktop

## Final Verification

Give the system a full reboot: sudo reboot.

The Chromebox will log in autonomously, sit on the desktop for 10 seconds while Docker initializes the layout, and then snap open into a beautiful full-screen digital signage loop. You can now connect to it via AirPlay at any time or manage your slides by pulling up the device's IP address on your laptop!